Notes on NCDG85.ZIP

GC-CAM can read G85 (slot drill) commands but converts them to simple routs.  This PDF can

be used to output them again as G85 commands, with the following limitations:

1)  If there are mixed slots and true routs, they must be placed on different layers and

output with different PDFs.  The CAM operator must do this manually.

2)  The PDF will only handle slots which don't connect end-to-start with other slots.

Happily this describes most slots.   Slots which do connect end-to-start need to

be manually modified so that they connect start-to-start or end-to-end.  That is

to say the PDF only works if each slot requires a move to the slot startpoint.