ECTSSXY.PDF

This plotter definition file will output a file for the Scorpion HV90SMT

system.  This PDF was created with GC-CAM version 2.2.

The form of the file created is

IMAGE: 1

X12345Y12345  X00100Y00100 1;

Where the first coordinate pair is the XY of the probe tip touching

the board under test and the second pair is the grid location.

"1" means that the test point is on the top surface.

";" Denotes an end of a network.

An extra semicolon is created at the beginning of the data file.

The PDF defaults to using the USER (0,0) coordinate system.  This

will enable you to use your error reports more effectively.

Note:  All probe tip location coordinates must be positive values!

Procedure:

1.  Create Test Fixture (F4)

2.  Create Grid and Map Layers (Alt-F4)

3.  Create Guide Plate(s) (Ctrl-F4)

4.  In the layer list rename the top MAP layer to "1" and

the bottom MAP layer to "0".  This is for the surface declaration.

5.  Write the ECT file from the map layer using ECTSSXY.PDF.

A. if you are doing flip testing on one fixture specify

the same output file name in two separate output fields.

Output File: TEST.XY

Layer:  1

Output File: TEST.XY

Layer:  2

This will append the flip test netlist information rather than

merge the information into the file.

B. If your tester is a clamshell, specify both map layers for one

output file.

Output File: TEST.XY

Layer:  1,2

5.  Upload the .XY into the ECT system.