# WinGPS Pro 3.3

- Manual EN

## (supplement)

### WinGPS Pro v3.3.1

March 27, 2003

© Copyright Stentec Software B.V. 2003

#### Introduction

This document is intended as a supplement to the existing manual of WinGPS Pro 3.3 and covers a number of new options that cannot be found in the existing manual. This manual is intended as a user’s manual and takes the user step by step through the new options. This document describes the options as they appear in WinGPS Pro v3.3.1. You can check the version number of WinGPS Pro by selecting Help -> Info in the menu. In the upper-left corner of the window you can read the build number. If you have WinGPS Pro version 3 but not version 3.3.1, then you can download the latest version from the internet. Visit http://www.stentec.com to download the latest version.

# 1 Garmin support

It is possible in WinGPS Pro to exchange data through the GARMIN protocol between a GARMIN device which supports the GARMIN protocol and WinGPS Pro. The following data can be transported (upload is from WinGPS Pro to the GARMIN device, download is from the GARMIN device to WinGPS Pro):

- Routes (up- & download)

- Waypoints (up- & download)

- Tracks (download only)

Before the following actions are performed, make sure your GARMIN device is set to the GARMIN/GARMIN protocol. How this is  done can be found in the manual of your device

## 1.1 Upload routes

- Choose the menu-option File -> Import/Export -> Garmin -> Upload routes.

- The list on the left shows all routes in the WinGPS Pro database. Click on one to select the route to be uploaded. Only one route can be uploaded at a time.

- Choose a routenumber. This is the number the route gets in your GARMIN device. Most GARMIN devices can contain multiple routes. Consult the manual of your device to learn more about the capabilities of your device.

- Choose a description for the route. This description is sent to the GARMIN device. Some devices require a unique description for each route. The description can contain 20 characters at most, however not all devices show the full 20 characters, other devices may even ignore the description. Consult the manual of your device to learn more about the capabilities of your device.

- The waypoints in a route are all labeled with a description. A GARMIN device doesn’t accept descriptions with more then 6 characters, so if a description is longer only the first 6 characters will be used. Also a GARMIN device expects unique descriptions for all waypoints. If all descriptions contain 6 characters or less, and if they are all unique, the route can be uploaded without a problem. If the descriptions contain more then 6 characters, auto numbering can be selected. Auto numbering uses the first 6 characters of a description, and replaces the last characters with an increasing number starting with 1. If desired a custom waypoint description can be entered, auto numbering is always on with this option.

- Click “Ok” to upload the route to the device.

## 1.2 Download routes

- Choose the menu-option File -> Import/Export -> Garmin -> Download routes

- Select the type of waypoint you want to store the routepoints in at “Select type”.

- Select the group you want to store the route in at “select group”.

- Select the Geodetic datum in which the routepoints are stored in the GARMIN device. Consult your manual to find out in what datum the routepoints are stored. You must select the correct geodetic datum, otherwise you can deviate while navigating.

- Click on “import” to download the routes. All routes in the GARMIN device are being downloaded separately and stored in the WinGPS Pro database. The routes are stored with the same name they have in the GARMIN device. If a route with the same name already exists in the database, it will be overwritten by the new one. Once stored in the database the name can be changed any time.

## 1.3 Upload (Proximity-) waypoints

- Choose the menu-option File -> Import/Export -> Garmin -> Upload (proximity-) waypoints.

- Choose in the left column the waypoint groups you want to upload by clicking on it. You can choose multiple groups by holding down the Control or Shift key while clicking on a group.

- Choose in the right column the waypoint types you want to upload. Initially all types are selected, so all waypoints will be uploaded.

- Waypoint descriptions in WinGPS Pro can contain up to 50 characters. A GARMIN device only accepts waypoint descriptions of 6 characters or less, and each description must be unique. If all waypoints in the selected groups are of 6 characters or less, and each name is unique, the waypoints can be uploaded without a problem. Otherwise auto numbering must be enabled. Auto numbering uses the first 6 characters of a description, and replaces the last characters with an increasing number starting with 1. If desired a custom waypoint description can be entered, auto numbering is always on with this option.

- Click “Ok” to upload the waypoints.

A GARMIN device can contain a limited number of waypoints. Consult your manual to see how much waypoints can be stored in your device.

## 1.4 Download (proximity-) waypoints

- Choose the menu-option File -> Import/Export -> Garmin -> Download (proximity-) waypoints

- Select the type of waypoint you want to store the waypoints in at “Select type”.

- Select the group you want to store the waypoints in at “select group”.

- Select the Geodetic datum in which the waypoints are stored in the GARMIN device. Consult your manual to find out in what datum the waypoints are stored. You must select the correct geodetic datum, otherwise you can deviate while navigating.

- Click on “Import” to download the waypoints. All waypoints in the GARMIN device will be downloaded and stored in the WinGPS Pro database with the same name as in the GARMIN device. If there is a waypoint in the WinGPS Pro database with the same name, it will be overwritten by the new one. Once stored in the database the name can be changed any time.

## 1.5 Download tracks

- Choose the menu-option File -> Import/Export -> Garmin -> Download tracks.

- Select the Geodetic datum in which the waypoints are stored in the GARMIN device at “Datum” . Consult your manual to find out in what datum the waypoints are stored. You must select the correct geodetic datum, otherwise you can deviate while navigating.

- If “Import as one track” is checked, all tracks will be merged to one single track, if not all tracks will be downloaded separately.

- Click “Import” to download the tracks.

- When the tracks are downloaded, an alternative description can be entered. To do so, select the track to change. It’s name will appear at “description”, which can be edited. To save the changes, click “Apply”. If desired, another track description can be changed.

2 How to import BSB charts

Since 2003 Maptech distibutes a lot of their European charts in the BSB 4.0 format. These charts can only be imported into WinGPS Pro after they have been registered. To register these charts, please read the instructions on the registration form or visit www.maptech.com.

To import BSB charts, start WinGPS Pro. Insert the CD-ROM with the charts into your CD-ROM drive. Select the menu option File -> Import/Export -> Import BSB charts. This will start the BSB Import wizard.

###### Enter searchpath

In the first window you have to enter the path where WinGPS Pro should search for BSB charts that you want to import. By pressing the ‘…’ button a window will be opened where you can select the searchpath. When you have selected the searchpath, press the ‘Select’ button. Press ‘Next’ to continue.

###### Select chartdatum

The next window shows all found BSB charts. The folowing information of each chart will be displayed: the filename, the datum as reported by the BSB chart, the datum used by  WinGPS Pro and the chartname and chartsetname. Since WinGPS Pro is unable to use the BSB chartdatum, you have to select the correct WinGPS Pro datum for each chart. Press the ‘Select datum’ to select a datum for the selected charts (if you hold down the Ctrl button while clicking a chart, you can select more than one chart at once).

All charts that are checked, will be imported. If WinGPS Pro finds charts that cannot be imported, then WinGPS Pro will show them in red. Press ‘Next’ to continue.

Copy charts

Now you have to decide wether to use the charts from the CD-ROM, or to copy them to your harddisk. If you want to leave them on CD-ROM, you always have to make shure the CD-ROM is in your drive when you want to use the charts.

Press ‘Next’ to continue.

###### WinGPS Pro folder

Now you have to choose a WinGPS Pro folder to put the charts in. Select one from the list, or enter a new one. If you check ‘do store in a folder’, the charts will not be placed in a folder. Press ‘Start’ to import the charts.

# 3 Digital Wave Atlas support

It is possible in WinGps Pro to use the Digital Wave Atlas IJsselmeer from Alkyon. To make this possible you must have the Digital Wave Atlas Cd Rom in your Cd Rom player and perform the following steps:

3.1 Installation

The Digital Wave Atlas must be installed first before it can be used. If you want to be able to use the Digital Wave Atlas even without having the Cd Rom in the Cd Rom player you can copy it to the harddisk. To install the Digital Wave Atlas choose the menu-option File -> Import/Export -> Import chart information file. Search for the WinGPS Pro installation folder in the upcoming dialog. In the root of this folder you will find a file named “golfatlas.chi”. Select it by clicking it and click “Open” to install the Digital Wave Atlas. To complete the installation, the path to the charts must be entered. Choose the menu-option Tools -> Wave Atlas. The installation path must be entered at “Installation path”. For example, if you want to read the charts from Cd Rom, you must enter the path to your Cd Rom player (for example “d:\”). If you copied the files to the harddisk, enter the path you copied the Cd Rom to. Click “Ok” to confirm.

3.2 Using the Digital Wave Atlas

The Digital Wave Atlas behaves like a normal navigation chart, with an option to set a few extra parameters. The charts are placed into two folders, “Golfatlas Ijsselmeer” and “Golfatlas Markermeer”. These can be found between the other chart folders. To set the extra parameters choose the menu-option Tools -> Wave Atlas. Three parameters can be set here: Wind speed, Wind direction and parameter (Wave height, length, steepness or refractive index). Depending on these parameters, WinGPS Pro will automatically select the correct chart.

4 Auto Import

Since the 2001 edtion of the DKW charts is WinGPS Pro notified when charts have been installed. WinGPS Pro 3.3 has support for this feature. Upcoming editions and updates will also use this feature to notify WinGPS Pro.

- 4.1 Selecting files

When there are new charts installed, WinGPS Pro will open the Auto Import window.

The list in the upper part of the window contains all new CHI files found by WinGPS Pro. These are calibration files used by WinGPS Pro to calibrate charts.

By default all files are selected. By using the left mouse button you can (de-)select one ore more files. Use the Control key in conjunction with the left mouse button to select or deselect (toggle). Use the Shift key in conjunction with the left mouse button to select or deselect a range of files. Click ‘Select all’ to select all files.

4.2 Import

One  mouse click on the ‘Import’ button, will let WinGPS Pro import all selected files into the database. The next time the program starts, it will not ask to import these files anymore. When not all files are selected, WinGPS Pro will ask to import the remaining files the next time it starts up.

##### 4.3 Ignore

When you do not want to import (some of) the selected files you can click the ‘Ignore’ button. Then WinGPS Pro will not import the selected files and will not ask for it anymore when it starts up. You can always manually import these files by selecting Files -> Import/Export -> Import chart information file from the menu.