# WinGPS Pro 3.3

- Manual EN

WinGPS Pro v3.3.1

March 27, 2003

© Copyright Stentec Software B.V. 2003

Table of Contents.

WinGPS Pro 3.3	1

Welcome to WinGPS Pro.	5

About this manual.	6

Getting started.	7

System requirements.	7

Installing WinGPS Pro.	7

Select a language.	7

Install WinGPS Pro 3.3	8

Create a diskette version.	8

Installing from diskettes.	8

Let’s get going.	9

Start WinGPS Pro.	9

A simulated trip.	9

Select the chart.	9

Waypoints.	10

Plan the route.	10

Sail the route.	10

WinGPS Pro at work.	13

The database.	13

Charts.	13

Bitmap scans.	13

Scanning a chart.	13

Calibrating a chart.	14

Folders.	15

The GPS receiver.	16

Connecting the GPS to the computer.	16

Configuring  the communication.	16

The GPS receiver.	16

The serial port of the computer.	16

Testing the communication.	17

Using the GPS receiver.	18

Creating waypoints.	18

Creating routes.	19

Sailing a route.	20

Tracking.	22

Extra information.	22

Creating the information database.	22

Entering information into the database.	23

The autopilot.	24

Connecting the autopilot to the computer.	24

Steering towards a waypoint.	24

Steering a course.	24

Connecting instruments.	25

Calling other ships.	25

Connecting a modem.	25

Configuring the modem.	26

Call other ships.	26

Simulate communication.	27

Importing data from the shareware versions.	27

Charts.	27

Waypoints, routes, tracks.	28

Service and support.	29

Customer service.	29

Technical support.	29

Stentec Software.	29

# Welcome to WinGPS Pro.

WinGPS Pro is the successor of the already very popular shareware program WinGPS.

WinGPS Pro is a program to assist you in your navigational tasks. WinGPS Pro can calculate tracks, distances and positions. You can enter waypoints into a database and these waypoints can be linked to create routes. Waypoints and routes can be plotted on a tracksheet or on a chart.

The most important facility of WinGPS Pro is the ability to use charts. These charts are bitmap scans of original charts that you can make yourself or that you can purchase from some map merchants. WinGPS Pro can plot tracks, routes and waypoints on the charts. Additionally WinGPS Pro provides the facility to draw accurate bearings, routes and zones directly onto the chart using the mouse.

You can enhance your charts by adding your own information like notes or photographs to particular positions. Furthermore you can mark dangerous or forbidden areas on your charts.

When you connect a GPS receiver to your computer, WinGPS Pro will read NMEA sentences from the GPS receiver about the current position, track, speed, date and time. This information is shown by WinGPS Pro in the status bar at the bottom of the screen. The current position and the track can be plotted on the chart and WinGPS Pro will warn you whenever you enter a zone that is marked as dangerous or forbidden. Finally you can instruct WinGPS Pro to keep a log of your trip.

As well as the GPS receiver you can connect other instruments via the NMEA interface to WinGPS Pro. With the appropriate instruments connected WinGPS Pro will show the pressure, wind, depth and water temperature. If you have a radar connected and that radar sends NMEA information WinGPS Pro will show the Electronic Bearing Marker and the so-called ‘tracked targets’ on the chart. WinGPS Pro can even control your autopilot.

If you connect your computer via a modem to a telephone WinGPS Pro can call other WinGPS Pro systems that are online. After a connection has been made WinGPS Pro will retrieve the position, course and speed of the called ship. After this has been done you can plot the other ships on your chart so you can see where they are and how they are proceeding.

WinGPS Pro uses a relational database to store all its information. You can use this information in other programs too. With Access or Excel you can create your own reports and make your own import and export routines.

Stentec Software, Heeg, the Netherlands publishes WinGPS Pro in several languages.

## About this manual.

Here is the idea behind this manual. Instead of simply telling you about the various features, we want to show you how to do things. That is why there are so many pictures in this manual. And that is why the manual is organised around tasks you will actually do, rather than around the program itself. We also wanted to keep this manual short, friendly and to the point.

Of course, there are drawbacks to this approach. The information you need might not be in the manual. Fortunately, WinGPS Pro comes with a very detailed Help system where you can find answers to questions not answered in this manual.

# Getting started.

## System requirements.

WinGPS Pro requires a system that runs under Windows 95/98 or Windows NT.

The minimum hardware required to run WinGPS Pro is:
•	Processor Pentium 133 MHz,
•	32 MB RAM,
•	VGA monitor (640x480).

To be able to use scanned bitmaps with a high resolution you will need a system equipped with at least a fast Pentium II processor, 64 Mb RAM and a SVGA monitor (800x600).

## Installing WinGPS Pro.

Insert the WinGPS Pro CD in your CD-ROM drive. The installation process should start automatically:

If nothing happens after you inserted the CD you may have Auto-Run disabled on your machine. No problem, just:

Double-click My Computer on your desktop,

Double-click your CD-ROM drive,

Double-click Setup.exe.

### Select a language.

First of all select the desired language by clicking on the related national flag.

### Install WinGPS Pro 3.3

Clicking on this button will start the installation program. If you have already installed a previous version, the installation program will upgrade the program files to version 3.3 but your navigational data will not be copied over. After installation you will still have the charts, waypoints, routes, etc. from your previous version.

Note.	If you upgrade from a previous version of WinGPS Pro, the installation program will not replace your existing database(s). This is done to preserve any information in your WinGPS Pro database(s).
The consequence is that you cannot join us on our simulated trip (see page 9). But we feel that an experienced user of previous versions does not need that introduction anymore.

### Create a diskette version.

If you need to install WinGPS Pro on a machine that is not equipped with a CD-ROM drive, you can create a diskette version of the installation files by clicking on this button. You will need 7 or 8 HD diskettes (1.44 MB).

### Installing from diskettes.

If your computer is not equipped with a CD-ROM drive you can install WinGPS Pro from the diskettes you created on a computer with a CD-ROM drive:

Insert diskette nr. 1 in the diskette drive,

Double-click My Computer on your desktop,

Double-click on 3.5-inch diskette,

Double-click on Setup.exe

# Let’s get going.

## Start WinGPS Pro.

Click on ‘Start / Programs / WinGPS Pro / WinGPS Pro 3.3’ and you will get:

For the moment, ignore the database. We’ll discuss that topic later. Click on Ok and the program will be loaded.

WinGPS Pro always starts with an empty chart (a tracksheet) that is centered at your last known position. An open red arrow indicates this so-called ‘current position’.

## A simulated trip.

To get a first impression of the main functions of WinGPS Pro, we invite you to join us on a simulated trip. WinGPS Pro comes with a folder that contains a set of demo charts and we can simulate the GPS.

Let’s assume that we are nearing the end of a passage from Lowestoft on the English East Coast to Den Helder in the Northwest of Holland. It’s raining and visibility is poor. So we’d better plan beforehand our route up the rather narrow Schulpengat, the southern approach to the port of Den Helder, and navigate by GPS.

### Select the chart.

Select from the menu bar the pull-down menu ‘Folders’ and click on ‘Select folder’. You’ll get a list of available folders. Double-click on ‘Zeegat van Texel’. Then go in the menu bar to ‘Charts’ and select ‘Search chart’. WinGPS Pro will extract from the folder the chart where we are now and display it on the screen. You’ll see your current position indicated by an open red arrow about a mile west of Verkenningston SG, which marks the entrance of the Schulpengat.

### Waypoints.

Let’s first see which waypoints have already been defined. Select from the menu bar ‘Waypoints’/ ‘Select waypoints’. You’ll get a list of available waypoint groups. Double-click on ‘Zeegat van Texel’ and the waypoints will be shown on the chart. There seem to be enough waypoints to plan the route to Den Helder.

### Plan the route.

Go to the menu bar and select under ‘File’ the function ‘Edit’ / ‘Routes’. Click in the window ‘Update routes’ on ‘New’. Give your route a name and click in ‘Update routes’ on ‘Waypoints’. Now you’ll get a list of the waypoints that have already been defined:

You select the waypoints of your route one by one in the left column and transfer them to the route by clicking on ‘Add >>’. While you are building your route, WinGPS Pro will keep track of the total distance in the lower right corner of the window. When you’re finished click on ‘Ok’ to save the route. Go in the menubar to ‘Routes / Select route’. Select the route Schulpengat > Den Helder and a green line on the chart displays the route. Close the ‘Update routes’-window when you’re finished.

### Sail the route.

First of all start the simulated GPS. Select ‘Communication’/ ‘Simulate communication’. This will bring up the ‘Simulation’ window:

Your current position is already filled in, but in this simulation you’ll have to provide the course and the speed yourself.

Now let’s see which course we have to steer. Select ‘Waypoints’/ ‘Go to waypoint’:

Move this window so you can see both the ‘Simulation’ and the ‘Go to’ window.

Select under ‘Tracks’ the functions ‘Start tracking’ and ‘Show track on chart’ so you can see the track you actually sailed afterwards.

Now enter the course, 90° magnetic in the ‘Simulation’ window, click on ‘Ok’ and watch your ship sailing on the chart. If you’re not very patient it might be a good idea to multiply your ship’s speed by about three. Six knots is a nice speed out at sea but it is horribly slow on a computer screen.

When you’re within half a mile from the waypoint you’ll get an audible alarm. Time to watch your ship on the chart. When you reach the waypoint, click on ‘Next’ in the ‘Go to’ window, enter the new course in the ‘Simulation’ window and click on ‘Adjust’.

Note.	Any adjustments made in the ‘Simulation’ window will become effective only after you click on ‘Adjust.

In this way you can sail your entire route. When you come level with buoy S7, WinGPS Pro will automatically switch to the next chart in the folder so that you also have the chart with the most space ahead of you.

While you’re sailing you‘ll see on the chart the planned route, your present position and the track you sailed.

- The green line is your planned route,

- The red line is your actually sailed track,

- The dotted line is the course from the last point where you changed course to the next waypoint.

If you want to save the track for later analysis you can do so by selecting the function ‘Save track’ under ‘Tracks’ in the menu bar.

It might be a good idea to make a few more simulated passages. It’s an excellent way to get accustomed to the program before you take WinGPS Pro out to sea.

# WinGPS Pro at work.

## The database.

WinGPS Pro stores all its information in a relational database. The active database is displayed on start-up of the program (see page 9).

WinGPS Pro comes with one database: ‘Gps.mdb’. Normally this will do. You can however create other databases. Under the ‘File’ menu there is an option ‘Create WinGPS database’. This will create an empty WinGPS Pro database. On start-up of the program you can switch between the databases by double clicking on the name of the database.

Note.	The WinGPS Pro database contains all your information. So it is important to save frequently a backup copy of your database on diskette(s). Do not simply copy the database; it might have outgrown the capacity of a single diskette. Instead use the Windows program ‘Backup’.

The WinGPS Pro database is actually a Microsoft Access 97 database. So you can access the database from outside WinGPS Pro in Access 97/2000 or Excel 97/2000.  Be very careful in doing so, the database can easily be damaged.
WinGPS Pro 3.3 can also handle Access 2000 databases.

## Charts.

- One of the great features of WinGPS Pro is the ability to work with bitmap scans of original paper charts. The copyrights on those charts allow you to make copies for your own use only. There is one exception: Stentec Software in Heeg is authorised to sell digital charts of the series 1800 published by the Dutch Dienst der Hydrografie van de Koninklijke Marine. All other charts you have to scan yourself or have them scanned by a professional copying service.

- WinGPS Pro 3.3 also supports the digital BSB charts published by Maptech. Stentec Software is an authorised dealer of BSB charts. NKF/Eurosea charts are also supported by WinGPS Pro 3.3. These charts are also available at Stentec Software.

### Bitmap scans.

WinGPS Pro works exclusively with charts in Mercator projection.

You could use charts in another cylindrical projection but the results will be less accurate. The deviation will increase with the distance from the reference points used to calibrate the map (see below). You cannot use charts in a stereographic or in another gnomic projection.

#### Scanning a chart.

Charts have to be scanned aligned very accurately with north up.

Do not use hand scanners; always use a flatbed scanner. It is virtually impossible to make good scans with a hand scanner; they are nearly always misaligned. The same applies to digital cameras.

Scanning charts can best be done with a resolution between 75 and 150 dots/inch. Smaller resolutions will give insufficient detail and larger resolutions will make the bitmap file too large for WinGPS Pro to handle smoothly.

The same applies to colours: use 16 colour scans (8 bits) and not 256 colour (16 bits).

Try to keep the bitmap files as small as possible. Large bitmap files will slow down WinGPS Pro considerably. It is better to have several small scans than to stitch multiple scans together.

Apart from the size of the resulting bitmap we consider stitching multiple scan together bad practice. It is hard enough to scan one page and keep it aligned. To stitch two scans together and keep them aligned is almost impossible.

WinGPS Pro accepts some twenty bitmap formats. See the online help for the list of the supported formats. We recommend that you use a format that compresses the bitmaps, like GIF or JPG.

#### Calibrating a chart.

After you have scanned a chart, you must calibrate it before you can use it in WinGPS Pro. To calibrate a chart, start the ‘Calibrate bitmap’ function from the ‘File’ menu. Click on the ‘Load bitmap’ button and select the bitmap you want to calibrate. The chart is then displayed in the ‘Calibrate bitmap’ window. Next you have to specify two clearly marked reference locations on the chart. The two locations should not have the same latitude or longitude. Try to make the differences in latitude and longitude as large as possible.

Now get the latitude and longitude of your reference points from the original paper chart or from another source. Select ‘Pos 1’, click on the first reference point in the bitmap and enter latitude and longitude. Then select ‘Pos 2’, click on the second reference point in the bitmap and enter latitude and longitude.

If you calibrate a bitmap of a non-Mercator chart you have to specify a third reference point – ‘Pos 3’. Be careful! The result will be less accurate than the calibration on two reference points of the scan of a Mercator chart.

Finally select ‘Common’. Here you enter a ‘Description’ (= name) for the calibrated chart and the chart ‘Datum’ that is printed on the original paper chart (on most charts in the legend or title box). Be sure to enter the correct chart datum. A wrong chart datum may cause serious errors in navigation.

Note.	In the list of chart datum’s is a datum called ‘Default’. This is not a real chart datum. It only indicates that WinGPS Pro does not have to do any datum conversion. It is meant for people who sail in an area where all charts have the same chart datum and who have set their GPS to the that datum.

After you press ‘Save chart’ the reference position, scale, descriptive name of the chart, chart datum and the filename of the bitmap are written to the database. The program will also calculate the minimum and maximum co-ordinates of the chart and record this information in the database too.

When the bitmap is not accurate enough WinGPS Pro will give a message. This may occur if the scanned chart was not Mercator projection, the scanning was not accurate or when the co-ordinates you use have a datum other than the datum of the chart. The chart can still be saved and used but may be not very accurate. You can check this by reading the co-ordinates, which WinGPS Pro shows when you move the mouse over the chart.

### Folders.

WinGPS Pro offers the possibility tot put related charts into folders, for instance adjacent charts or small/medium scale charts of coastlines and large-scale charts of harbours and approaches. The big advantage of using folders is that WinGPS Pro can automatically select the chart that matches your position and course.

Select in the menu bar ‘File / Edit / Folders’ and click in the window ‘Update folders’ on ‘New”. Enter a name for the new folder and then click on ‘Charts’. You select the charts one by one in the left column and put them into the folder by clicking on ‘Add >>’.

You set the way WinGPS Pro will select charts in the window ‘File / Setup / Navigation / General’. ‘Folder search’ gives you two possibilities:

- ‘Most space’ looks for the chart, which offers the most space in the direction you are sailing.

- ‘Largest scale’ looks for the most detailed chart.

## The GPS receiver.

WinGPS Pro communicates with the GPS receiver via a serial port of the computer. The program will expect from the GPS receiver NMEA-183 sentences. So your GPS must be able to send those sentences. Check this in the documentation of the GPS first.

### Connecting the GPS to the computer.

You need a cable that connects the GPS receiver to (one of) the serial port(s) of your computer. This cable should connect:

- •	NMEA-B	to  data-in	(pin 2) of the serial port,
•	NMEA-A	to  data-out	(pin 3) of the serial port,
•	GPS ‘ground’	to  ground	(pin 5) of the serial port.

For almost any GPS receiver you can buy a cable to connect the GPS to the serial port of a computer. Some GPS receivers require a special cable. Always consult the documentation of your GPS to see what cable is needed.

### Configuring  the communication.

#### The GPS receiver.

Make sure that the GPS is set to NMEA-send. The documentation of your GPS will tell you how to do this.

#### The serial port of the computer.

Start the Setup program from the File menu. Choose ‘Communication’. This will bring up a configuration window:

The values shown are the default values. The defaults will be suitable for most GPS-receivers. Consult the documentation provided with your GPS-receiver to see if these values have to be adjusted.

WinGPS Pro 3.3 supports up to 4 serial ports. You can connect and disconnect your computer's ports to WinGPS Pro 3.3 by means of the buttons ‘Add’ and ‘Delete.

Check the ‘NMEA output’ checkbox to configure the port to send NMEA-messages.

Check the ‘Autopilot’ checkbox if you will connect an autopilot to this port.

Note.	WinGPS Pro does not use the values in the Windows configuration.

### Testing the communication.

Switch on your GPS and select in WinGPS Pro from the menu bar:

- ‘Communication / Start communication’,

- ‘Communication / Show Messages’.

You will get a window where the incoming NMEA sentences are displayed.

You don’t get any messages? Check:

- Is your GPS switched on?

- Do you use the right cable?

- Is the cable correctly connected?

- Is the serial port correctly configured – did you specify the right port?

- Is the COM port used by an intern modem?

- Is the interrupt (IRQ) of the COM port used by another device?

Correct the error and test again.

### Using the GPS receiver.

Each time you want to use your GPS receiver with WinGPS Pro you have to start the communication by ‘Communication / Start communication’.

You can stop the communication with the GPS receiver any time by ‘Communication / Stop communication’.

## Creating waypoints.

There are three ways to create waypoints:

- Go to ‘File / Edit / Waypoints / New’ and enter latitude and longitude by hand.

- Click with the right mouse button on the position of the waypoint in the chart and click in the pop-up menu on ‘Mark mouse position’.

- Select in the waypoint menu the option ‘Mark current position’. In this way you can store your current position as a waypoint for future use.

In all cases you will get a window where you have to supply some additional information:

This is the window you get from ‘File / Edit / Waypoints / New’. The other two methods will give you a similar window with the latitude and longitude already entered.

‘Type’ is used to identify the type of waypoint on the chart: light, red buoy, route point, etc. You can specify an icon for each waypoint type; this icon is displayed on the chart.

With ‘Group’ you can divide your waypoints into groups. You could make a waypoint group for each chart, for a certain trip and the like.

‘Datum’ is the chart datum on which the entered latitude/longitude are based.

You must specify an existing type and group. Therefore you have to create the waypoint types and waypoint groups first in the menu items:

‘File / Edit / Waypoint types / New’
and
‘File / Edit / Waypoint groups / New’

Note.	WinGPS Pro comes with a series of predefined waypoint types. You can edit these types to suit your own preferences by:
‘File / Edit / Waypoint types / Edit’

After you have created waypoints, you can display the waypoints on the chart by ‘Waypoints / Select waypoints’ in the menu bar.

When you move the cursor over a waypoint on the chart, the cross will change into a little hand. Clicking with the left mouse button will show all data of that waypoint.

Note.	There is a fourth way to create waypoints. If you draw a route on the chart, WinGPS Pro will automatically create waypoints for all the breakpoints in the route (see below). The program will place all these waypoints in a new group that gets the name of the route. All waypoints get the type ‘Route point’.

## Creating routes.

There are three ways to create routes:

- Draw the route on the map.

Put the cursor in the chart on the point of departure. Hold down the [Alt] key and double-click. The cursor changes into a small pencil. Use that pencil to draw your route. Click with the left mouse button where you want to change course. When you reach the end of the route double-click on the point of arrival. WinGPS Pro will ask for a name and stores the route under that name.

Note.	Instead of double clicking in combination with the [Alt] key you can also click the right mouse button and then choose the ‘Draw route’ function from the popup menu.

- Compile the route from existing waypoints.

Select in the menu bar ‘File / Edit / Routes’ and click in the window ‘Update routes’ on ‘New”. Enter a name for the new route and then click on ‘Waypoints’. You select the waypoints of your route one by one in the left column and transfer it to the route by clicking on ‘Add >>’.

While you are building your route, WinGPS Pro will keep track of the total distance in the lower right corner of the window.

- Convert a previously stored sailed track to a route.

Select in the menu bar ‘Tracks / Track to route’:

Specify the track that you want to convert and enter a name for the new route.

The maximum ‘XTE Nm’ will result in a route that deviates never more than ‘XTE Nm’ miles from the previously sailed track – this could be a route with a large number of waypoints!

The maximum ‘Waypoints’ will produce a route that approximates the previously sailed track by the specified number of waypoints. Check very carefully whether the new route stays clear of all obstacles along the route!

When you click on ‘Select’ the route is displayed on the chart.

To display an already existing route on the chart you go to ‘Routes / Select route’ in the menu bar. If you check ‘Reverse route’ the route will be loaded in the opposite direction.

## Sailing a route.

There are several ways to sail a planned route:

- Print the route by the option ‘File / Print / Routes’ and give the list of waypoints, distances and courses to be steered to the helmsman.

- You could use the printed route to program the route in your GPS and read the GPS to do your navigation.

- You could upload your route from WinGPS Pro to the GPS receiver. For the details consult the ‘Help’ for ‘File / Import/Export’.

- Select the route you want to sail. Then select the option ‘Waypoints / Go to waypoint’ from the menu bar. This will bring up the ‘Go to’ window:

This function always starts on the first waypoint of the selected route. If you are in or past this position, click on ‘Next’ to get the values for the next leg. These values are updated every 5 seconds. You can adjust this interval by the function ‘File / Setup / Navigation / ‘General’.

What happens when you reach the next waypoint depends on the set-up of the ‘Go to’ function. Select ‘File / Setup / Autopilot/Goto:

If you check ‘Alarm at arrival’ you will get an audible signal at ‘Alarm at’ Nm from the waypoint. If you check ‘Turn automatically’ the ‘Go to’ window will switch to the next waypoint as soon as you are ‘Turn at’ Nm from the waypoint.

If you have connected an autopilot to WinGPS Pro, you can check ‘Autopilot’. WinGPS Pro will then instruct the autopilot to steer towards the waypoint. We will discuss the autopilot later (see page 24)

Note.	You can also use the function ‘Go to waypoint’ apart from routes:

If you have previously selected waypoints you can select a waypoint as the destination. Click on ‘Start’ to activate the function.

If you invoke the function by clicking the right mouse button in the chart on the destination and select ‘Goto mouse position’ in the pop-up menu, the mouse position will become the destination and the function will start automatically.

## Tracking.

You can instruct WinGPS Pro to keep track of your trip. Select in the menu bar ‘Tracks / Start tracking’. WinGPS Pro will then continuously plot your current position. You can specify the interval of plotting in ‘File / Setup / Navigation / General’.

When you check also ‘Tracks / Show track on chart’ the entire track of your trip will be drawn on the chart.

If you want to review your trip afterwards, you can save the track by selecting ‘Tracks / Save track’.

Note.	You can also draw tracks on the chart by hand.  This seems a bit illogical but it is a nice facility in planning a trip. Double-click on the starting point. The cursor will change into a pencil and you can draw a possible route. Click with the left mouse button where you want to change course. While you are drawing the bearing, the distance along the current leg and the total distance are continuously shown in the upper left corner of chart. Double clicking will stop the drawing and erase the track.

Instead of double clicking on the starting point of you trip you can also use the right mouse buttons and then chooses ‘Draw track’ from the popup menu.

## Extra information.

WinGPS Pro offers you the possibility to attach your own information to positions on your charts. This information can be any document (text, html, photographs, sound, video) of which the document-type is registered in Windows. WinGPS Pro stores a reference to the document together with the position in an information database. The documents itself must be created beforehand outside WinGPS Pro by a Windows program.

### Creating the information database.

The function ‘File / Create info database’ will create an empty information database:

In the empty form you must fill in the first field. It will become the name of the Info database. The other three fields are optional. Click on the button ‘Create’ to create the database.

### Entering information into the database.

First you have to open the database by the function ‘File / Open info database’ and select the right database (you can create more than one info database). You will get a window that is similar to the one above. Click on the button ‘Open’ to open the database. When ‘Show info on chart’ is checked small icons on the chart will represent all information. You can also show the information on the chart afterwards by the function ‘Charts / Show info’.

Now put the cursor on the position to which you want to attach your information, click with the right mouse-button and fill in a name of the position and the full path to the file you want to attach to this position. If ‘Show info’ is activated you will see a small icon after clicking the button ‘Ok’.

When you move the cursor over an information icon it will change into a small hand to indicate that you can click to see the information:

Note.	If you get the message ‘File not found’ after you clicked on the icon, you have probably misspelled the name of the file that you wanted to attach to this position. Click with the right mouse button on the icon and use the function ‘Edit info’ to correct the mistake.

## The autopilot.

WinGPS Pro can instruct your autopilot to:

- steer towards a waypoint,

- steer a magnetic course.

- steer a course relative to the apparent wind.

Warning.	This facility is still in beta. It has been tested with an Autohelm ST4000 autopilot and it seems to function properly. At the moment of the release of version 3.3 there were no test-results with other autopilots available.

### Connecting the autopilot to the computer.

The autopilot should have its own magnetic compass and no connection to the GPS receiver. If the autopilot is attached to an on-board network, this connection should be removed otherwise the functions in WinGPS Pro will not function properly.

For further instructions on how to connect and operate your autopilot, please read the owners manual of the autopilot.

### Steering towards a waypoint.

This facility requires only the GPS receiver to be connected to WinGPS Pro.

If you select the function ‘Waypoints / Go to waypoint’ from the menu bar and you have checked the box ‘Autopilot’ in the ‘Setup / Autopilot/Goto’ window (see page 20), WinGPS Pro will control the autopilot to steer towards the waypoint.

### Steering a course.

This facility requires the appropriate instruments to be connected to WinGPS Pro (see page 25).

Select ‘Tools / Autopilot’ in the menu bar. The program will show the magnetic heading and the relative wind.

You can lock the autopilot on one of these values. With the << and >> keys you can adjust the locked value by 10 degrees, with the < and > keys you can adjust the locked value by 1 degree.

To lock the autopilot on the magnetic course the program needs to receive the magnetic heading (HDM or VHM sentence), to lock the autopilot on the relative wind, the program needs to receive the wind direction (VWR sentence).

When locked on the relative wind you can use the ‘Tack’ button to tack the ship, the new locked relative wind direction will be the same as the current one but with the opposite sign.

## Connecting instruments.

The way you can connect the instruments aboard to WinGPS Pro varies widely from one manufacturer to another. You will have to dig into the documentation that came with your instruments to find out.

If you have an on-board network there is usually a central point that you can connect to the serial port of your computer.

If you have several instruments that are not part of a network you will need more than one serial port. WinGPS Pro 3.3 supports up to 4 serial ports.

## Calling other ships.

WinGPS Pro has the option to call another ship that also uses WinGPS Pro. After a connection has been made WinGPS Pro programs will retrieve the position, course and speed of the other ship. After this has been done you can plot the other ship on your chart so you can see where it is and how it is proceeding.

To use these options both WinGPS Pro systems have to be equipped with a modem and a telephone.

### Connecting a modem.

As one serial port is already occupied by the cable to the GPS receiver, you will have to connect the modem to a second serial port. If your computer has only one serial port (like all notebooks) you will need an internal modem or a PCMCIA modem card.

### Configuring the modem.

Start the Setup function from the File menu. Choose ‘Modem’. This will bring up a configuration window:

When you set ‘Receive calls’ to ‘Yes’; WinGPS Pro will answer all incoming telephone calls. You cannot be reached by voice and you cannot call other ships. Other ships can call you to retrieve your position, course and speed.

When you set ‘Receive calls’ to ‘No’, WinGPS Pro will ignore incoming telephone calls. Now you can be reached by voice and you can call other ships. Other ships cannot call you to retrieve your position, etc.

A new value of ‘Receive calls’ (‘Yes’/‘No’) takes effect only after you exit WinGPS Pro and restart the program.

All other default values will probably be suitable for your modem. Consult the documentation of your modem to make sure.

### Call other ships.

Before you can call other ships you have to enter their names and telephone numbers in a phone book.

Go to ‘File / Edit / Ships’. You will get an empty phone book. Click on ‘New’ and enter for each ship you want to call the name and telephone number.

To call a ship make sure that ‘Receive calls’ is set to ‘No’ in the modem set-up and go to ‘Tools / Call ships’. WinGPS Pro will display the names of the ships in the phone book. Select the ship you want to call and click on ‘Call’. After the connection has been made, WinGPS Pro gets information about course, speed, position and wind from this ship. This information is then stored in your database.

‘Tools / Plot ships’ will display an icon with the name for each ship on the last received position. When you click on an icon WinGPS Pro will display the last received position, course, speed and wind of the corresponding vessel.

‘Tools / Show ships’ displays a table with all ships and their last received data.

## Simulate communication.

This is a very powerful tool to explore the possibilities of WinGPS Pro. In fact it simulates a ship equipped with a GPS receiver.

The function ‘Communication / Simulate communication’ will give you the ‘Simulation’ window:

You must enter the position where you start your simulated trip, your course and your speed. When you click on the tab ‘Environment’ you can also specify the direction and the speed of the apparent wind. After clicking on ‘Ok’ your virtual ship starts sailing and WinGPS Pro will simulate the input from the GPS and other instruments.

While you are sailing you can change the course, the speed and the wind. The altered values take effect when you click on ‘Adjust’. In this way you can make simulated passages and watch WinGPS Pro do his job.

Note.	A boat speed that results in a fast passage in the real world, is horribly slow when you simulate the passage on your computer. Do not hesitate to specify a speed that is a multiple of your real ship’s performance.

## Importing data from the shareware versions.

### Charts.

Put your bitmap files (*.BMP, *.GIF, etc) and chart files (*.CHT) into the chart folder of WinGPS Pro.

For each chart open the ‘Calibrate bitmap’ window (see page 14) and load the chart file instead of the bitmap. WinGPS Pro will store the calibration data in your database.

### Waypoints, routes, tracks.

To import waypoints, routes and tracks from your shareware version use the function ‘File / Import/Export / WinGPS’. WinGPS Pro will store the information from the waypoint files (*.WP), route files (*.RT) and track files (*.TRC) in its database.

# Service and support.

## Customer service.

For all non-technical questions about Stentec products, including price information, upgrades, etc. please contact us:

- On our web-site http://www.stentec.com

- By e-mail: software@stentec.com

- By fax: +31 515 442 824

- By phone: +31 515 443 515 from 09:00 to 17:00 West European time.

## Technical support.

If you encounter technical problems that you cannot solve by consulting this manual and the online help:

- Look on our website http://www.stentec.com/support/WinGPS Pro You might find an answer to your question in there.

- Send your problem by e-mail to helpdesk@stentec.com . Include as much information on the problem as possible. State also your hardware and your operating system – Windows 95, Windows 98 or Windows NT.

- Send your problem by fax to +31 515 442 824

- Phone the helpdesk on +31 515 443 515 between 09:00 and 17:00 West European time.

## Stentec Software.

Hollingerstraat 14

8621 CA  Heeg.

The Netherlands.