Release Notes

| Program: FBH2200E | Type of Change: Driver (Hardware, firmware, driver, XPA. Etc) Release Date: 2011/09/19 | Revision Number: 5.72.00919’11 Replaced Revision: 5.60.0223’11 Creation Date: 2011/09/19 |
| --- | --- | --- |
| Author or Designer: George |  |  |
| What Changed? |  |  |
| What has been Fixed? | FBH2200E.ds - Modify for Bank of Communications in China. - Show scanning done message box in non-UI mode when setting "CloseUIDialog" in advui.ini or scanner.ini to 1. - To upgrad firmware for HP Scanjet series. - Correct code mistake. - Do not reset PageCx, PageCy to avoid wrong selection area on UI. - Get maximum resolution of endorser from ini. - Output OS informtion log. - GammaOut 180 as default. - To define/specify interpolating resolution for all resolution from ini. - Support to specify the inserted resolution only by interpolating. - Support CAP_FEEDERALIGNMENT. - Function improvement of CAP_DEVICEONLINE. - Modify for new cmatch operation. - Do not reset value of ICAP_FRAMES while getting capability for Inspector TWAIN only. - Limited the maximum resolution to 600 dpi for Inspector TWAIN. - Modify to get usb vid pid. - Fixed inspector TWAIN failure when ReserveForFirmware is not zero in NVRam. - Define ResInch in ini to get resolution from dots per inch. - Return the resoultion unit from inch with "Microtek Scanner Component" - Save the current scan mode while receiving scan mode changed. - Avoid to add over scan length while cropping with x, y offset are not zero. - Enable application window in DisableDS(). - Add continue button in transferring timeout dialog. - Compare the maximum width from ADF and flatbed and set the bigger one to ICAP_FRAMES for Inpsector TWAIN only.Add continue button in transferring timeout dialog. - To get overscan length if ability was supported. - To skip or determine message MSG_PROCESSEVENT from "DSEVENT" in ini. - Modify capability ICAP_BRIGHTNESS for "OMR SCAN PRO". - The exception case that measured unit of resolution by inch not ICAP_UNITS for Inpsector TWAIN. - Custom capabilities for FU-500. - Enable long page mode for FU-500. - Modify code for the permission of everyone. - Support new capability ICAP_SPLITIMAGE and splitting image settings. - Support new capabilitys CAP_POWERSWITCHCOUNT, CAP_ADFLIGHTINGTIMEFRONT, CAP_ADFLIGHTINGTIMEREAR, CAP_FLATBEDLIGHTINGTIME for FU-500. - Modificaion skew and imprinter control for FU-500. - Modification for segmental compression of JPEG. - Return driver detected error before scanning. - New capability CAP_GETFALLBACK. - Do not reset y offset in long page mode even if y offset is over the first length. - Bug fixed long page setting in flatbed mode. - Bug fixed the confilct setting of ScanMethod (HIWORD) in AbilityEX. Advance.dll - Read parameter EndorserMaxRes from ADVUI.ini to determine MaxRes value if scanner support MICR or ENDORSER or IMPRINTER FUNCTION. - Color Matching add Mix, Custom Items. - add Front / Rear IR Scan in Image Tab. - Add AutoCropCheckTop for Automatic - Check cropping. - 重新計算 x-offset, 避免稅務專案的 default.av2 裏的 x-offset 在 A3 scanner 會出現錯誤(此 x-offset 值由 A4 機種得到) - 當 background =1 (黑背景) 時, 如果切換到固定尺寸, 介面上維持設定黑背景, 不做更改. - 修正選擇自訂 profile 且有開 color match 功能, UI 會去檢查 ICC profile 是否存在, 不存在需把 color match 功能關閉. - new features : (1) Live preview with custom drop out (2) Edge Extend(邊緣延伸) (3) Split Image(圖像分割). - 修改 long page 的 unknow length 字串, 顯示為第二長度. - 解決 Win7 64-bit 系統下當 registry 裏的 DSLOC 不存在時 LoadLibrary advance.dll 會失敗. - fix bug that show "long page" size on flatbed scan. - Modified adjusted x-offset or y-offset should not change whidth or length. - 修正 scanner 不支援 flatbed scan 時, scan source 仍會顯示 "automatic", "flatbed". Twdsm_n.dll - Change the location of log in GetADFStatus(). - Avoid executing function GetADFStatus multiple times while in scanning. - Output IR scan log. - Marked this statement #pragma argsused for compile warning. - Fixed GetDeviceList is no use since 1.3.1.85 and disable GetDeviceList manually. - Do not call TuneScanLength in fixed size without over scan and check cropping. - To send all possible dirction for TuneScanLength even if overscan is 0 line. - New parameters for GeneralOperaion. - Modify for segmental compression of JPEG. - Import/Export GetFallback function. AVIP.dll - Add DoEdgeExtend function. - Fixed the formula error in DoEdgeExtend. - Fixed the formula error of lineart in DoEdgeExtend. Avscan_n.dll - No separation processing with flatbed cropping while gNoEOP to 1. - Full page cropping with flatbed mode. - Support to create compressed raw data of G4 and Jpeg and erase all debug images in the first scanning. - Output inserted pixel image in debug mode. - Output blank and memory data. - Modification for gamma 1.8. - Do not send endorser command in flatbed. |  |
| Known Defects |  |  |
| Required Files And Revision Level | File Name | Revision |
|  | FBH2200E.dll | 0.9.2011.915 |
|  | Advance.dll | 1.3.0.47 |
|  | SmartImage.dll | 1.0.108.0 |
|  | FBH2200E.ds | 2.1.1.2 |
|  | Twdsm_n.dll | 1.2.1.96 |
|  | AVIP.dll | 0.0.0.43 |
|  | Avscan_n.dll | 1.3.2.3 |
|  | fpcreator_img.dll | 1.0.0.617 |
|  | AVglowing.dll | 1.0.49.0 |
|  | Cmatch.dll | 4.3.7.0 |
|  | AVText.dll | 11/3/7 |
|  | AvCropEx.dll | 0.5.10.0 |
|  | AVBlank.dll | 0.1.0.3 |
|  | gray_quality.dll | 1.0.0.5 |
|  | AVScanDoubleSizeLib.dll | 1.3.0.1 |
|  | LDyIp.dll | 2.0.1.11 |
|  | AVBarcode.dll | 1.0.0.5 |
|  | AVJpegRAWToJFIFLib.dll | 1.2.0.14 |
|  | AVJpegEncoder.dll | 1.1.0.4 |
|  | Av2PDF.dll | 1.0.0.2 |
|  | ColorDropOut.dll | 2005/8/26 |
| Details: |  |  |

Test Overview:

Scanner Driver Info & Test Setup

Enter information about the scanner/driver test.

| Scanner Model Name: | FBH2200E | I/O driver version: | 0.9.2011.214 |
| --- | --- | --- | --- |
| Firmware Version: | 0.10 | Application: | Photoshop 7.0 |
| TWAIN driver version: | 5.60.0223’11 | Tester: | Erin |
| Test Date: | 2011/02/23 |  |  |

| # Installer Testing |  |  |
| --- | --- | --- |
| Step | WinXP | Comments |
| - Install |  |  |
| - Digital signature inside | OK |  |
| - Model name information | OK |  |
| - Uninstall |  |  |
| - Model name information | OK |  |
| - Driver remove | OK |  |
| - Without reboot message (USB) | OK |  |

| # Basic Functionality Testing |  |  |
| --- | --- | --- |
| Step | WinXP | Comments |
| - Simulate Mode Test |  |  |
| - A4, 300, B/W (Dynamic threshold) | OK |  |
| - A4, 200, Gray (Document) | OK |  |
| - A4, 100, Color | OK |  |
| - A4, 300, Gray (Photo) | OK |  |
| - A4, 200, Color (Color - Document) | OK |  |
| - A4, 100, B/W (Fixed Processing) | OK |  |
| - A4, 300, Color (Color - Photo) | OK |  |
| - A4, 200, B/W (Halftone 1) | OK |  |
| - A4, 100, Gray | OK |  |
| - Button Test |  |  |
| - Scan button (Scan Led 1) | OK |  |
| - Email button (Scan Led 2) | OK |  |
| - Print button (Scan Led 3) | OK |  |
| - Cancel button | OK |  |

| # Special Function Testing |  |  |
| --- | --- | --- |
| Step | WinXP | Comments |
| - Software Image Processing |  |  |
| - Corp+Deskew (Color) | OK |  |
| - Crop (Gray) | None |  |
| - Blank Page Detection | OK |  |
| - Rotation 90 CW degree | OK |  |
| - Color drop - R | OK |  |
| - Custom drop out | OK |  |
| - Multi-feed Detection | None |  |
| - Ultrasonic Detection | None |  |
| - Intelligent Mode | None |  |
| - Ultrasonic Area | None |  |
| - Mirror | OK |  |
| - Edge Fill | OK |  |
| - Despeckle | OK |  |
| - Power Saving | OK |  |
| - Cache Memory Size | None |  |
| - Cache None | None |  |
| - Prefeed | None |  |
| - Transport Timeout | OK |  |
| - Digital Imprinter | OK |  |

| # Adverse Conditions Testing |  |  |
| --- | --- | --- |
| Step | WinXP | Comments |
| - Jamming Test | None |  |
| - Cancel Scan Test | OK |  |
| - ADF Cover Open | None |  |