SIMATIC

S7-200 PC Access V1.0 SP6

Copyright (c) Siemens Industry Inc., 2011

Contents

Notes on installation

# 1	Operating system and hardware requirements

# 2	Operating environment

# 3	Installation

Notes on Usage

# 4	What's new

# 5	Known problems and remedies

5.1	Problems

5.2	National languages

5.3	General communications

5.4	Multi-master communications

5.5	Shutdown procedure

Notes on installation

1	Operating system and hardware requirements

- Microsoft Windows XP SP3 (32-bit only) or Windows 7 (32-bit and 64-bit versions).

- A personal computer (PC) capable of running one of the above operating systems, refer to Microsoft specifications for more information.

- At least 150 Mbytes of free hard disk space

- Use the small font setting and a minimum screen resolution of 1024x768 pixels

- Any mouse supported by Microsoft Windows

- For communicating with the S7-200 you will need one of the following:

–	PC/PPI Cable connected to your USB port

–	PC/PPI Cable connected to your serial communications port (PC COM1 or COM2)

–	Communications processor (CP) card and multipoint interface (MPI) cable

–	Multipoint interface (MPI) card and communications cable

–	EM241 modem expansion module

–	CP243-1 or CP243-1 IT Ethernet expansion module

- CP card support for PC Access V1.0 SP6 on Windows XP SP3 (32-bit only) and  Windows 7 (32-bit and 64-bit):

- CP5512	supported

- CP5611	supported
The Set PG/PC Interface device CP5611(PPI) is not supported when installed on Windows 7 64-bit. Users may choose the CP5611(MPI) or CP5611(PROFIBUS) as an alternative. The CP5611(MPI) interface may be used for all baud rates expect 9.6kbs. The CP5611(PROFIBUS) interface may be used for all baud rates.

- CP5613	not supported

- CP5614	not supported

- PG/PC interface setup with Windows 7 64-bit operating system

The format of the device names in the 64-bit version of the PG/PC interface has changed from the previous 32-bit PG/PC interface that is described in the system manual and Help topics. In the 64-bit version, the device names take the format of: <Device.Protocol.InstanceNumber>, where “Device” is the name of the device such as "PC/PPI cable", Protocol is the communications protocol such as "PPI" or "MPI", and “InstanceNumber” is a number from 1..n (typically 1, unless you have multiple devices of the same type installed).

- Installation problems with SIMATIC NET
If you have SIMATIC NET V6.2 or earlier installed on you computer you may receive the following error during installation.  
"Fatal error in SIMATIC NET Component sntieno.dll"
"Installation of protocol siem_isotrans failed with error code hr=0x80070430"

If you experience this error follow these steps:
1. Finish the installation and reboot your PC.  Rebooting is required.
2. Use Add/Remove Programs to uninstall the software and reboot your PC.  Rebooting is required.
3. Install the software again, this time you will not receive the error.

2	Operating environment

To be able to install and work with S7-200 PC Access software under Windows XP, you must have logged on with at least power user privileges.

To be able to install and work with S7-200 PC Access under Windows 7, you must have logged on with administrator privileges.

It is recommended that you disable hibernation mode when using S7-200 PC Access. Entering hibernation mode while online connections are open can cause the following errors:

- Unintentional termination of online connections

- Data loss in any open projects opened through a PC network

To avoid these errors, set the energy saving options for the operating system in the Windows Control Panel to manual trigger, so that hibernation is not automatically triggered by a timer. Do not manually trigger hibernation mode during a communication task, as described above.

S7-200 PC Access Version 1.0 SP6 has been tested under the following operating system versions:

- Windows XP (XP Professional with SP3, 32-bit version only)

- Windows 7 (32-bit and 64-bit versions)

S7-200 PC Access Version 1.0 SP6 has been tested with the Microsoft Excel plug-in for Excel versions 2003 and 2007.

Only STEP 7-Micro/WIN V4.x releases can accommodate a simultaneous connection with S7-200 PC Access v1.x.  For instance you cannot run communications in STEP 7-Micro/WIN V3.x and S7-200 PC Access v1.x at the same time.

Note: If you experience unstable operation or unexplained program lockups, please examine the BIOS settings of your computer and make sure hyper-threading is disabled.  To resolve other situations please reboot your PC before running the application again.

3	Installation

1. Close all applications including the MS-Office Toolbar.

2. Insert the S7-200 PC Access CD into the CD-Rom drive. The Setup program will run automatically if you have not disabled the auto start option. You can manually start the Setup program by double clicking the file "Setup.exe" located on the CD.

3. Follow the instructions displayed to complete the installation.

Note: To install S7-200 PC Access on the Windows XP or 7 operating system, you must log in with administrator rights.

Notes on usage

4	What's new

To find out what's new in V1.x, run the S7-200 PC Access application and double-click on the "What's New" link at the top of the project tree.

5	Known problems and remedies

5.1	Problems

1.	Help topics describing Microsoft Windows operation

The PC Access V1.0 SP6 Help system contains graphics and text that describe operation with Microsoft Windows versions and communication devices that are no longer supported. Refer to the “Operating system and hardware requirements” section of this readme file for operating system and communication support information.

2. 	Installing CP Cards

You should always install S7-200 PC Access before inserting and installing any CP card. S7-200 PC Access contains I/O driver software that is used by CP cards.

3.	Managing Applications Access to the Serial Port

If you use the S7-200 PC Access Communications Setup or the PC/PG Interface under the PC Control Panel to set up the PC COM PORT parameters, certain conditions will cause the COM Port selection field to be shown with an asterisk beside the COM Port number. This asterisk indicates that the user's PC setup has already assigned another application to use the same COM Port. For example, if the Palm Pilot software is installed on the PC and you try to install S7-200 PC Access, this asterisk will appear because Palm Pilot software typically loads each time the PC is booted up. You must unload (not uninstall) the other application to free up the COM Port so that S7-200 PC Access can gain access to the port.

4. 	TC35 Cell Phone Modems

Certain Dell laptops used with a Siemens TC-35 modem will not properly communicate. The problem is seen with some Dell laptops that drive the transmit line high when the serial port is not in use. Because of this, the Siemens TC-35 (and possibly other industry standard modems) resets causing initialization sequencing problems followed by communication errors.

5. 	TC-35 Cell Phone Modem Remote Programming

Remote modem programming of a Siemens TC-35 modem uses an &W AT-command. Early firmware versions of this modem do not support this command. Programming of this modem will fail, if this command is not supported.

- Special Characteristics of the MS Windows 7 Operating System

Note that the software in MS Windows 7 is no longer under "Start menu > SIMATIC"; it is now located under "Start menu > Program Files > Siemens Automation > SIMATIC". This change is necessary due to the current architecture of MS Windows.

5.2	National languages

S7-200 PC Access uses several Microsoft common dialog boxes. The text in these dialogs appear in the national language of the operating system, regardless of the language you have chosen for S7-200 PC Access.

5.3	General communications

Refer to the S7-200 Programmable Controller System Manual located on the Documentation CD for a list of communication configurations.

5.4	Multi-master communications

Note:  When using Multi-Master communications on networks with several masters (including TDs or OPs), the hardware takes several seconds to initialize onto the PLC network. If communication is attempted before this initialization is complete, an error may occur. Retrying the last request should correct it.

Setting S7-200 PC Access network addresses:

As a general rule, use the lower available addresses for masters and the higher available addresses for slaves. It is preferable, but not required, to use consecutive addresses for the masters on a network. For example:

## Network components	Possible addresses

S7-200 PC Access	0

TD 200	1

OP15	2

Several S7-200 slaves	3, 4, and 5 You could leave these numbers free for additional masters and start the numbering of the S7-200 slaves at 16.

5.5	Shutdown procedure

In environments where PC Access is forced to shut down, it is strongly suggested to use a mechanism to inform remote clients of the server’s removal.  Failure to inform the remote clients properly could result in errors when restarting. The recommended shutdown process is to set a value in the registry prior to shutting down, which will send a broadcast shutdown message to all clients.

Using "HKEY_LOCAL_MACHINE\SOFTWARE\Siemens\SIMATIC\OPC\S7-200 PC Access"

1. Set DWORD value ForceShutdown to 1 (setting to 1 forces a shutdown of the OPC Server after the time in the key SecondsShutdown. The OPC Server sends a shutdown message to all connected clients with the text in the key ReasonShutdown)

2. (optional) Set String value ReasonShutdown to reason (contains the shutdown message).

3. Set DWORD value SecondsShutdown to 0 (contains the time between shutdown message and the shutdown of the OPC Server).

For example, the script causing the shutdown would first set the ReasonShutdown to "Good-bye" (or something), set SecondsShutdown to 0, and set ForceShutdown to 1.  Now the script can call something like shutdown -f.  PC Access and the OPC Server would have shut down normally and sent the shutdown message to all clients before stopping.  The system will shutdown properly after handling the OPC handshaking and restart with no issues.

PC Access should be shut down properly. If an improper shutdown occurs due to a power failure or system crash, then additional steps may be required (on restart) to notify the clients that the server was reset (before proper communication can be restarted).